EXP NO: 2 -   CHARACTERISTICS STUDY OF SIDE BAND POWER AND FREQUENCIES IN AM USING MATLAB

Objective 1:

Program code:

clc; clear; close all;

Am = 1;

fm = 500;

Ac = 5;

fc = 5000;

fs = 20 * fc;

t = 0 : 1/fs : 4/fm;

m  = Am * sin(2 * pi * fm * t);

c  = Ac * sin(2 * pi * fc * t);

mu = Am / Ac;

am = Ac * (1 + mu * sin(2 * pi * fm * t)) .* sin(2 * pi * fc * t);

dem = abs(hilbert(am));

dem = (dem - mean(dem)) / Ac;

N      = length(am);

f      = linspace(-fs/2, fs/2, N);

AM_FFT = abs(fftshift(fft(am))) / N;

USB = fc + fm;

LSB = fc - fm;

Pc  = (Ac^2) / 2;

Psb = (mu^2 * Pc) / 4;

figure('Name', 'AM Modulation Analysis', 'NumberTitle', 'off');

subplot(3, 2, 1);

plot(t * 1e3, m, 'b', 'LineWidth', 1.5);

title('Message Signal');

xlabel('Time (ms)'); ylabel('Amplitude');

grid on;

subplot(3, 2, 2);

plot(t * 1e3, c, 'r');

title('Carrier Signal');

xlabel('Time (ms)'); ylabel('Amplitude');

grid on;

subplot(3, 2, 3);

plot(t * 1e3, am, 'k');

title('AM Modulated Signal');

xlabel('Time (ms)'); ylabel('Amplitude');

grid on;

subplot(3, 2, 4);

plot(t * 1e3, dem, 'm', 'LineWidth', 1.5);

title('Demodulated Signal');

xlabel('Time (ms)'); ylabel('Amplitude');

grid on;

subplot(3, 2, 5);

plot(f, AM_FFT, 'b', 'LineWidth', 1.2);

xlim([fc - 3*fm, fc + 3*fm]);

title('Spectrum (Sidebands Around fc)');

xlabel('Frequency (Hz)'); ylabel('|X(f)|');

grid on;

subplot(3, 2, 6);

bar([Pc, Psb, Psb], 0.4, 'FaceColor', [0.2 0.6 0.8]);

set(gca, 'XTickLabel', {'Carrier', 'USB', 'LSB'});

title('Power Distribution');

ylabel('Power (W)');

grid on;

fprintf('u = %.2f | USB = %d Hz | LSB = %d Hz | Pc = %.2f W | Psb = %.3f W\n', mu, USB, LSB, Pc, Psb);